Computer Organization and

Architecture – Assessment Answers Based on the uploaded assessment question paper.

1. Addressing Modes

Given: PC = 4200H, R1 = 1500H, R2 = 2500H, IX = 0800H, SP = 3000H. Operand = 200H.

Memory: [1500H]=45H, [1502H]=3200H, [2500H]=18H, [2800H]=3400H, [3000H]=3600H, [3200H]=55H, [3400H]=66H, [3600H]=77H, [4204H]=200H.

Immediate: EA = Not applicable (operand is part of instruction). Operand value = 200H.

Direct: EA = 0200H. Operand value = M[0200H]. The value at 0200H is not supplied in the question.

Indirect: EA = M[0200H]. The memory content at 0200H is not supplied, so the final EA/operand cannot be determined from the given data.

Register: EA = Not applicable. If the register specified is R1, operand value = R1 = 1500H.

Register Indirect: If R1is used: EA = 1500H. Operand value = M[1500H] = 45H.

Indexed: EA = 0200H + IX = 0200H + 0800H = 0A00H. Operand value = M[0A00H], which is not supplied.

Relative: EA = PC + 0200H = 4200H + 0200H = 4400H. Operand value = M[4400H], which is not supplied.

StackAddressing: EA = SP = 3000H. Operand value = M[3000H] = 3600H.

Note: The supplied memory table does not contain all addresses needed for Direct, Indirect, Indexed, and Relative modes, so their final operand values cannot be obtained without inventing data.

b) Maximum memory accesses: Indirect addressing generally requires the maximum memory accesses because one memory access is needed to obtain the effective address and another to obtain the actual operand.

c) Increasing execution speed (typical order):

Indirect<Direct<Indexed/Relative<Register Indirect<Register<Immediate. Exact ordering can depend on the processor implementation.

2. Execution Time and Speedup

Given: Instruction count = 900 million = 900 × 10⁶, CPI = 2.8, original clock = 2.4 GHz, upgraded clock = 3.6 GHz.

1) Original execution time:

CPU cycles = 900 × 10⁶ × 2.8 = 2.52 × 10⁹ cycles.

T₁ = 2.52 × 10⁹ / 2.4 × 10⁹ = 1.05 seconds.

2) New execution time:

T₂ = 2.52 × 10⁹ / 3.6 × 10⁹ = 0.70 seconds.

3) Speedup = T₁ / T₂ = 1.05 / 0.70 = 1.5× (50% faster).

4) If CPI increases by 10%: New CPI = 2.8 × 1.10 = 3.08.

Revised cycles = 900 × 10⁶ × 3.08 = 2.772 × 10⁹ cycles.

Revised execution time = 2.772 × 10⁹ / 3.6 × 10⁹ = 0.77 seconds.

5) Increasing clock frequency is not always equally beneficial. With unchanged CPI, time decreases from 1.05 s to 0.70 s. When CPI rises by 10%, the time is 0.77 s, so the improvement is smaller. Therefore, clock frequency should be considered together with CPI and other processor factors.

3. Instruction Mix and Performance

Instruction mix: Arithmetic = 5 × 10⁶ instructions, CPI 2; Memory = 3 × 10⁶, CPI 5; Branch = 1 × 10⁶, CPI 4; Floating Point = 2 × 10⁶, CPI 8. Clock = 2.5 GHz.

1) Total clock cycles:

Arithmetic = 5 × 10⁶ × 2 = 10 × 10⁶

Memory = 3 × 10⁶ × 5 = 15 × 10⁶

Branch = 1 × 10⁶ × 4 = 4 × 10⁶

Floating Point = 2 × 10⁶ × 8 = 16 × 10⁶

Total = 45 × 10⁶ cycles.

2) CPU execution time = 45 × 10⁶ / 2.5 × 10⁹ = 0.018 s = 18 ms. 3) Total instructions = 11 × 10⁶ . Average CPI = 45/11 = 4.09 approximately.

4) New memory cycles = 3 × 10⁶ × 3 = 9 × 10⁶ . New total cycles = 10 + 9 + 4 + 16 = 39 × 10⁶ cycles.

New execution time = 39 × 10⁶ / 2.5 × 10⁹ = 0.0156 s = 15.6 ms.

5) Performance improvement = (Old time − New time)/Old time × 100 = (18 − 15.6)/18 × 100 = 13.33%.

4. Processor Comparison

Formula: CPU time = Instruction Count × CPI / Clock Frequency. Processor A: (500 × 10⁶ × 2.5)/(4 × 10⁹ ) = 0.3125 s.

Processor B: (550 × 10⁶ × 1.8)/(3 × 10⁹ ) = 0.33 s.

Processor C: (430 × 10⁶ × 3.1)/(5 × 10⁹ ) = 0.2666 s approximately. 2) Ranking from fastest to slowest: C (0.2666 s)<A (0.3125 s)<B (0.33 s).

3) Speedup of fastest C over slowest B = 0.33 / 0.2666 ≈ 1.238×, or about 23.8% faster.

4) Processor B with 10% fewer instructions: New instruction count = 550 × 10⁶ × 0.90 = 495 × 10⁶ .

New time = (495 × 10⁶ × 1.8)/(3 × 10⁹ ) = 0.297 s.

5) Recommendation: Processor C is the best overall choice for this workload because it has the lowest execution time (about 0.2666 s). Its higher clock frequency and lower instruction count outweigh its higher CPI. Processor A is second, while B is slowest before the instruction-count reduction.

5. Memory Trace Problem

Initial memory: M[2500H] = 15H and M[2501H] = 25H.

Assuming standard accumulator operations where ADD B performs A ← A + B: 1) Execution trace:

LDA 2500H → A = 15H, B = unchanged.

MOV B,A → B = 15H, A = 15H.

LDA 2501H → A = 25H, B = 15H.

ADD B → A = 25H + 15H = 3AH, B = 15H.

STA 2502H → M[2502H] = 3AH; A = 3AH, B = 15H.

HLT → Execution stops; A = 3AH, B = 15H.

2) Value stored at 2502H = 3AH (58 decimal).

3) Flags modified: The ADD instruction modifies the arithmetic/status flags. For 25H + 15H = 3AH, Carry = 0, Zero = 0, Sign = 0, and Parity = 1 in an 8-bit accumulator. The exact flag set depends on the processor architecture.

4) Compact execution trace:

Start → A=?, B=? → LDA 2500H → A=15H → MOV B,A → B=15H → LDA 2501H → A=25H → ADD B → A=3AH → STA 2502H → M[2502H]=3AH → HLT.

Final Answers – Quick Review

Q2: Original time = 1.05 s; New time = 0.70 s; Speedup = 1.5× ; Revised time with 10% CPI increase = 0.77 s.

Q3: Total cycles = 45 × 10⁶ ; Time = 18 ms; Average CPI ≈ 4.09; New time = 15.6 ms; Improvement ≈ 13.33%.

Q4: A = 0.3125 s; B = 0.33 s; C ≈ 0.2666 s; Ranking C>A>B; C over B ≈ 1.238× speedup; B after10% instruction reduction = 0.297 s.

Q5: Final A = 3AH, B = 15H, M[2502H] = 3AH.