COMPUTER ARCHITECTURE
CO1 – ASSESSMENT TOOL 1 & ASSESSMENT TOOL 2

Merged Answer Document

PART I – CO1 ASSESSMENT TOOL 1 ANSWERS

The first paper contains five application/numerical questions. The answers below preserve the values and conclusions from the prepared CO1_AT1 answer document.

Q1. Addressing Modes

Given: PC = 4200H, R1 = 1500H, R2 = 2500H, IX = 0800H, SP = 3000H; operand = 200H.

Memory: [1500H] = 45H, [1502H] = 3200H, [2500H] = 18H, [2800H] = 3400H, [3000H] = 3600H, [3200H] = 55H, [3400H] = 66H, [3600H] = 77H, [4204H] = 200H.

Immediate: EA not applicable because the operand is part of the instruction; operand value = 200H.

Direct: EA = 0200H; operand = M[0200H]. The value at 0200H is not supplied.

Indirect: EA = M[0200H]. The memory content at 0200H is not supplied, so the final EA/operand cannot be determined.

Register: EA not applicable. If R1 is used, operand value = 1500H.

Register Indirect: using R1, EA = 1500H; operand = M[1500H] = 45H.

Indexed: EA = 0200H + 0800H = 0A00H; M[0A00H] is not supplied.

Relative: EA = 4200H + 0200H = 4400H; M[4400H] is not supplied.

Stack: EA = SP = 3000H; operand = M[3000H] = 3600H.

Maximum memory accesses: Indirect addressing generally needs the most because one access obtains the effective address and another obtains the operand.

Typical increasing execution speed: Indirect < Direct < Indexed/Relative < Register Indirect < Register < Immediate. Exact ordering depends on implementation.

Q2. Execution Time and Speedup

Instruction count = 900 × 10⁶; CPI = 2.8; original clock = 2.4 GHz; upgraded clock = 3.6 GHz.

CPU cycles = 900 × 10⁶ × 2.8 = 2.52 × 10⁹ cycles.

Original execution time = 2.52 × 10⁹ / 2.4 × 10⁹ = 1.05 s.

New execution time = 2.52 × 10⁹ / 3.6 × 10⁹ = 0.70 s.

Speedup = 1.05 / 0.70 = 1.5×, i.e., 50% faster.

If CPI increases by 10%: new CPI = 3.08; revised cycles = 2.772 × 10⁹; revised time = 0.77 s.

Conclusion: increasing clock frequency helps, but CPI and other processor factors must also be considered.

Q3. Instruction Mix and Performance

Arithmetic: 5 × 10⁶ instructions, CPI 2 → 10 × 10⁶ cycles.

Memory: 3 × 10⁶ instructions, CPI 5 → 15 × 10⁶ cycles.

Branch: 1 × 10⁶ instructions, CPI 4 → 4 × 10⁶ cycles.

Floating Point: 2 × 10⁶ instructions, CPI 8 → 16 × 10⁶ cycles.

Total clock cycles = 45 × 10⁶.

At 2.5 GHz, CPU execution time = 45 × 10⁶ / 2.5 × 10⁹ = 0.018 s = 18 ms.

Total instructions = 11 × 10⁶; average CPI = 45 / 11 ≈ 4.09.

If memory CPI becomes 3: memory cycles = 9 × 10⁶; new total = 39 × 10⁶ cycles.

New execution time = 39 × 10⁶ / 2.5 × 10⁹ = 15.6 ms.

Performance improvement = (18 − 15.6)/18 × 100 ≈ 13.33%.

Q4. Processor Comparison

Formula: CPU time = Instruction Count × CPI / Clock Frequency.

Processor A = (500 × 10⁶ × 2.5)/(4 × 10⁹) = 0.3125 s.

Processor B = (550 × 10⁶ × 1.8)/(3 × 10⁹) = 0.33 s.

Processor C = (430 × 10⁶ × 3.1)/(5 × 10⁹) ≈ 0.2666 s.

Fastest to slowest: C > A > B.

Speedup of C over B = 0.33 / 0.2666 ≈ 1.238×, about 23.8% faster.

Processor B with 10% fewer instructions: 495 × 10⁶ instructions; new time = 0.297 s.

Recommendation: Processor C is best for this workload because it has the lowest execution time.

Q5. Memory Trace Problem

Initial memory: M[2500H] = 15H and M[2501H] = 25H.

LDA 2500H  →  A = 15H

MOV B,A    →  B = 15H, A = 15H

LDA 2501H  →  A = 25H, B = 15H

ADD B      →  A = 25H + 15H = 3AH, B = 15H

STA 2502H  →  M[2502H] = 3AH

HLT        →  Execution stops

Final value stored at 2502H = 3AH (58 decimal).

For the ADD result 25H + 15H = 3AH: Carry = 0, Zero = 0, Sign = 0, Parity = 1 for an 8-bit accumulator; the exact flag set depends on the processor architecture.

PART II – CO1 ASSESSMENT TOOL 2 ANSWERS

The second paper is a concept-mapping assessment. The following are ready-to-draw concept maps in a neat text-arrow format.

Q1. CPU, Memory, I/O and Bus Structures

COMPUTER SYSTEM

│

┌─────────────────────┼─────────────────────┐

↓                     ↓                     ↓

CPU                  MEMORY                  I/O

│                     │                     │

┌────┼────┐            ┌───┴───┐           Input / Output

↓    ↓    ↓            ↓       ↓             devices

ALU  CU  Registers     Cache    RAM

│    │      │           │       │

└────┴──────┴───────────┴───────┴──────────────┐

│                         │

BUS ───────────────→ Data / Address / Control

│

┌──────────────┼──────────────┐

↓              ↓              ↓

Single-bus      Multi-bus      Hierarchical bus

shared path     parallel paths  multiple levels

│              │              │

more contention   less conflict   reduces traffic on main bus

│              │              │

└──────────────┴──────────────┘

↓

Performance / Bottlenecks

│

memory latency • bus contention • I/O delay • bandwidth

↓

Multi-bus / hierarchical bus

↓

higher parallelism & throughput

During execution, the Control Unit generates control signals, registers hold temporary data/instructions, the ALU performs arithmetic and logic, memory supplies/stores instructions and data, and I/O devices exchange information with the system.

Single-bus structures are simple but can suffer from contention. Multi-bus structures allow more simultaneous transfers. Hierarchical buses separate high-speed and lower-speed traffic, reducing congestion.

Q2. Instruction Execution Cycle and Performance

INSTRUCTION EXECUTION

│

↓

FETCH → DECODE → EXECUTE

│        │         │

│        │         ├── ALU / control / memory operation

│        │         └── result produced

│        └── identify opcode + operands

└── read instruction from memory

│

↓

CLOCK CYCLES / CPI

│

↓

CPU Time = Instruction Count × CPI / Clock Rate

│

┌────────┼───────────┐

↓        ↓           ↓

Memory access  Instruction  Pipeline stalls

latency        complexity   / hazards

│        │           │

└────────┴───────────┘

↓

EXECUTION TIME

│

↓

BENCHMARKING

│

compare workloads

↓

evaluate performance

Fetch contributes time through instruction-memory access; decode contributes through instruction interpretation and operand identification; execute contributes through ALU, control, memory, or I/O operations.

Higher CPI, memory delays, complex instructions, and pipeline stalls increase execution time. Benchmarking uses representative workloads to compare actual system performance.

Q3. 8085 vs 8086 Architecture

MICROPROCESSORS

│

┌────────┴────────┐

↓                 ↓

8085              8086

│                 │

8-bit processor     16-bit processor

│                 │

Registers: A,     General registers:

B, C, D, E,       AX, BX, CX, DX

H, L + flags      + SP, BP, SI, DI + segments

│                 │

simpler addressing   more addressing modes

│                 │

no memory            Memory segmentation:

segmentation         CS, DS, SS, ES

│                 │

smaller/simple     larger program space

│                 │

└────────┬────────┘

↓

PROGRAM EXECUTION

│

8086 supports more complex

programs and addressing

↓

efficiency / multitasking

8085 has an 8-bit architecture and a simpler register/addressing organization. 8086 has a 16-bit architecture, a larger register set and memory segmentation using CS, DS, SS and ES.

The richer 8086 architecture supports larger and more complex programs and can improve efficiency for applications requiring more data/address manipulation. Multitasking is primarily an operating-system capability, but processor features can support efficient task management.

Q4. Addressing Modes and Instruction Types

ADDRESSING MODES

│

┌───────┬────────┼────────┬────────┐

↓       ↓        ↓        ↓        ↓

Immediate   Direct  Register  Indirect  Indexed

│       │        │        │        │

operand in  address  operand   address   base/index +

instruction of data  in reg   in memory  displacement

│       │        │        │        │

└───────┴────────┴────────┴────────┘

↓

INSTRUCTION TYPES

│

┌─────────────┼─────────────┐

↓             ↓             ↓

Data transfer   Arithmetic    Control

│

↓

ACCESS / PERFORMANCE

│

fewer memory accesses → generally faster

more memory accesses → generally slower

shorter/simple forms → smaller instruction

│

↓

REAL-WORLD PREFERENCE

Immediate → constants / initialization

Register   → fast repeated calculations

Direct     → fixed memory variables

Indirect   → pointers / dynamic data

Indexed    → arrays / tables

Immediate addressing is fast because the operand is part of the instruction and usually avoids an extra operand-memory lookup.

Register addressing is efficient because the operand is already in a CPU register. Direct addressing is useful for fixed memory locations. Indirect addressing is useful for pointers and dynamic structures. Indexed addressing is especially useful for arrays and tables.

The exact instruction size and execution time depend on the processor/ISA implementation.

Q5. Binary, Hexadecimal, Instructions, Memory and Debugging

NUMBER SYSTEMS

│

┌────────┴────────┐

↓                 ↓

Binary           Hexadecimal

base 2             base 16

│                 │

└───────┬─────────┘

↓

INSTRUCTION / DATA

│

machine representation

↓

MEMORY

│

instruction bytes

↓

CPU

│

fetch → decode → execute

│

↓

DEBUGGING

│

inspect addresses / opcodes / data values

│

↓

hexadecimal is convenient

because 1 hex digit = 4 bits

Binary uses only 0 and 1 and directly represents the two-state logic used by digital circuits.

Hexadecimal uses 16 symbols (0–9 and A–F). One hexadecimal digit represents four binary bits, so long binary values can be written much more compactly.

Example: binary 1010 1111 = hexadecimal AF.

Instructions and data are stored as binary bit patterns in memory. The CPU fetches an instruction, decodes it, and executes it.

Hexadecimal is widely used in low-level programming and debugging because memory addresses, machine codes, and bit patterns are much easier to read than long binary strings.

Reference to the Papers

CO1 AT1 prepared answers: the merged document incorporates the existing answer set for addressing modes, execution time/speedup, instruction mix, processor comparison, and memory trace. fileciteturn3file0L11-L37

CO1 AT2 question paper: five concept-mapping questions cover CPU/memory/I/O and buses, instruction execution and performance, 8085 vs 8086, addressing modes, and number systems/debugging. fileciteturn3file1L124-L151