SIMATS ENGINEERING
Computer Architecture – Assessment Tool 1
Analytical Problem Solving – Answers

1. Signed 8-bit Addition and Subtraction: +45 and −23

Given: +45 and −23, using 8-bit 2’s complement representation.

Binary conversion:
+45 = 0010 1101
+23 = 0001 0111
To represent −23, invert 0001 0111 → 1110 1000 and add 1:
−23 = 1110 1001

Addition (+45) + (−23):
  0010 1101
+ 1110 1001
------------
1 0001 0110

Discard the carry out of the 8th bit:
0001 0110 = +22.

Therefore, +45 + (−23) = +22.

Subtraction (+45) − (−23):
Subtraction is performed by adding the 2’s complement of −23. The 2’s complement of 1110 1001 is 0001 0111, which represents +23.

0010 1101
+ 0001 0111
------------
  0100 0100

0100 0100 = +68.

Overflow detection:
For signed addition, overflow occurs when two operands with the same sign produce a result with the opposite sign. In both operations here, the result is within the 8-bit signed range (−128 to +127). Hence, no overflow occurs.

The processor detects overflow by examining the carry into and carry out of the sign bit, or equivalently by checking whether the operand signs and result sign indicate a signed overflow.

2. 4-bit Carry Look-Ahead Adder (CLA)

For each bit i, let Ai and Bi be the input bits.

Generate signal:
Gi = Ai · Bi

Propagate signal:
Pi = Ai ⊕ Bi

Carry equations for a 4-bit CLA:
C1 = G0 + P0C0
C2 = G1 + P1G0 + P1P0C0
C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0
C4 = G3 + P3G2 + P3P2G1 + P3P2P1G0 + P3P2P1P0C0

Sum:
Si = Pi ⊕ Ci

The important point is that the carries are computed directly from the generate/propagate signals rather than waiting for each carry to travel through the next full adder.

Comparison:
In a ripple carry adder, the carry must pass sequentially from bit 0 to bit 1, then bit 2, and so on. Therefore, delay increases approximately linearly with the number of bits.

In a CLA, the carry equations are formed in parallel using logic gates. This greatly reduces carry-propagation delay. For wider processors, CLA structures can be combined hierarchically to obtain faster addition.

Hence, CLA improves processor speed because carry computation is performed in advance/parallel instead of waiting for a ripple of carries through every bit.

3. Booth’s Algorithm: (−6) × (+5)

Use 4-bit 2’s complement numbers.

Multiplicand M = −6:
+6 = 0110
−6 = 1010

Multiplier Q = +5 = 0101

Initialize:
A = 0000
Q = 0101
Q−1 = 0
M = 1010

Booth rules:
Q0 Q−1 = 01 → A = A + M
Q0 Q−1 = 10 → A = A − M
Q0 Q−1 = 00 or 11 → No arithmetic operation
Then perform an arithmetic right shift on (A, Q, Q−1).

Iteration 1:
Q0Q−1 = 10, so A = A − M.
A = 0000 − 1010 = 0110 (equivalent to +6).
Arithmetic right shift:
A = 0011, Q = 0010, Q−1 = 1.

Iteration 2:
Q0Q−1 = 01, so A = A + M.
A = 0011 + 1010 = 1101.
Arithmetic right shift:
A = 1110, Q = 1001, Q−1 = 0.

Iteration 3:
Q0Q−1 = 10, so A = A − M.
A = 1110 − 1010 = 0100.
Arithmetic right shift:
A = 0010, Q = 0100, Q−1 = 1.

Iteration 4:
Q0Q−1 = 01, so A = A + M.
A = 0010 + 1010 = 1100.
Arithmetic right shift:
A = 1110, Q = 0010, Q−1 = 0.

Final product is AQ = 1110 0010.

11100010 is an 8-bit 2’s complement negative number. Its magnitude is 30, so the result is −30.

Therefore:
(−6) × (+5) = −30.

Booth’s algorithm is efficient because it examines transitions in the multiplier and can replace consecutive runs of 1s with fewer addition/subtraction operations. Arithmetic shifting preserves the sign, allowing signed multiplication to be handled naturally.

4. Restoring and Non-Restoring Division: 13 ÷ 3

Dividend = 13 = 1101
Divisor = 3 = 0011

Expected result:
13 ÷ 3 = quotient 4, remainder 1.

A. Restoring division

Use registers A (remainder), Q (dividend/quotient), and M (divisor).

Initial:
A = 0000
Q = 1101
M = 0011

The algorithm performs one left shift of (A,Q), subtracts M from A, and checks the sign. If A becomes negative, M is restored and the quotient bit is 0; otherwise the quotient bit is 1.

After four iterations, the quotient register becomes:
Q = 0100
and the final remainder is:
A = 0001.

Thus:
Quotient = 0100 = 4
Remainder = 0001 = 1.

B. Non-restoring division

The non-restoring method avoids immediately restoring A after a negative subtraction. Instead, the next iteration adds M when the partial remainder is negative and subtracts M when it is non-negative.

Starting with A = 0000 and Q = 1101, four iterations are performed. The sign of A determines whether M is added or subtracted in the next step. After the final correction, if A is negative, M is added once.

Final result:
Quotient = 0100 = 4
Remainder = 0001 = 1.

Comparison:
Restoring division may require an extra restore operation whenever a subtraction produces a negative partial remainder. Non-restoring division avoids this repeated restoration by deciding the next operation from the current sign of the partial remainder.

Therefore, non-restoring division generally requires fewer corrective operations and can be implemented efficiently in hardware. This is why it is often preferred for high-performance arithmetic units.

5. IEEE 754 Representation of −13.25

Decimal number = −13.25.

Step 1: Convert to binary.

13 = 1101
0.25 = 0.01

Therefore:
13.25 = 1101.01₂

Normalize:
1101.01₂ = 1.10101₂ × 2³

Sign bit:
S = 1 because the number is negative.

A. Single precision (32-bit)

IEEE 754 single precision contains:
1 sign bit + 8 exponent bits + 23 fraction bits.

Exponent:
Actual exponent = 3
Bias = 127
Stored exponent = 3 + 127 = 130
130 = 10000010₂

Fraction:
The normalized value is 1.10101₂, so the fraction field stores the bits after the leading 1:
10101000000000000000000

Thus the fields are:
Sign = 1
Exponent = 10000010
Fraction = 10101000000000000000000

The 32-bit pattern is:
11000001010101000000000000000000

B. Double precision (64-bit)

IEEE 754 double precision contains:
1 sign bit + 11 exponent bits + 52 fraction bits.

Actual exponent = 3
Bias = 1023
Stored exponent = 3 + 1023 = 1026
1026 = 10000000010₂

Fraction field:
1010100000000000000000000000000000000000000000000000

Thus:
Sign = 1
Exponent = 10000000010
Fraction = 1010100000000000000000000000000000000000000000000000

Floating-point addition:
1. Compare the exponents.
2. Shift the significand of the smaller exponent to align the binary points.
3. Add or subtract the significands according to their signs.
4. Normalize the result.
5. Adjust the exponent.
6. Round the fraction according to the selected IEEE 754 rounding mode.

Important issues:
• Normalization keeps the significand in the required form.
• Rounding is necessary when the exact result cannot fit in the available fraction bits.
• Precision loss can occur when a smaller number is aligned with a much larger number and significant bits are shifted out.
• Overflow or underflow may occur when the exponent goes outside the representable range.

IEEE 754 therefore provides a standard method for representing and processing real numbers consistently in computer systems.

End of Answers